## 3.2 Hierarchical 3D Scene Graph

To enable high-level reasoning, we construct a hierarchical scene graph $\mathcal{G}_{S}$ that abstracts the environment into room and object-level entities, grounded by a fine-grained navigational graph. The construction process consists of three stages: dynamic mapping, topological graph generation, and semantic classification.

### Dynamic RGB-D Mapping

The foundation of the scene representation is a semantic 3D map built from the robot's onboard perception.

- Voxel Grid Construction: The agent processes a stream of posed RGB-D frames $\{I_{0},...,I_{t}\}$ containing semantic information1. The point clouds from these frames are transformed into a global coordinate frame and arranged into a dynamic 3D voxel grid $\mathcal{M}_{t}$2. This grid is updated continuously as the agent explores new areas or observes dynamic changes3.

- Occupancy Map: To facilitate navigation, we derive a two-dimensional bird's-eye-view (BEV) occupancy map $\mathcal{B}_{t}$ from the 3D grid4. This is achieved by analyzing "stixels" (vertical columns of voxels) in $\mathcal{M}_{t}$; we identify the highest occupied entry per stixel to infer obstacle positions, walls, and explored free space $\mathcal{F}_{t}$5.

### Voronoi Graph (GVD)

We abstract the dense metric map into a sparse topological graph $\mathcal{G_{V}}$ to support efficient navigation and spatial reasoning.

- Computation: We first inflate the obstacles in the BEV map $\mathcal{B}_{t}$ using a Euclidean Signed Distance Field (ESDF) to ensure robustness6. Based on this inflated map, we compute a Generalized Voronoi Diagram (GVD)7. The GVD consists of a set of points (nodes) that have equal clearance to the closest obstacles8. We filter this diagram by excluding nodes that are in the immediate vicinity of obstacles or outside the explored bounds $\mathcal{B}_{t}$9.

- Navigation Utility: The resulting nodes and edges form the navigational Voronoi graph $\mathcal{G_{V}}=(\mathcal{V},\mathcal{E})$10. We extract the largest connected component to serve as the robot-centric navigation graph, sparsifying it to reduce the node count11. This graph allows the robot to navigate robustly by maximizing clearance from obstacles.

### Scene Graph & Room Classification

The final layer, the 3D Scene Graph $\mathcal{G}_{S}$, clusters the Voronoi graph into semantic regions (rooms) and assigns objects to them.

- Room Clustering (Door-Based Separation): Unlike previous methods that rely on geometric constrictions (which fail in open layouts), we separate the global Voronoi graph $\mathcal{G}_{\mathcal{V}}$ into distinct room regions $\mathcal{G}_{\mathcal{V}}^{R}$ based on detected doors12.

  - We model the probability distribution of observed door positions using a mixture of Gaussians: $\rho_{\mathcal{N}}(x,H)=\frac{1}{N_{D}}\sum_{i=1}^{N_{D}}K_{H}(x-x_{i})$, where $x_{i}$ are door coordinates and $H$ is the bandwidth matrix13131313.

  - Edges and nodes of the Voronoi graph that fall into high-probability door regions are removed, effectively cutting the graph into disjoint components representing distinct rooms14.

  - Connectivity between these rooms is re-established by calculating shortest paths between the disjoint components; if a path traverses exactly two components, they are marked as neighbors15.

- Object Assignment: Objects are assigned to rooms by minimizing a weighted travel distance. For an object $o$, we identify the Voronoi node $n_{o}$ that minimizes the path length to the viewpoint $v_{p}$ from which the object was seen, weighted by the Euclidean distance to the object (Eq. 2 in the paper)16161616. This prevents erroneous assignments through walls17.

- LLM-Based Room Classification: Once rooms are clustered and objects are assigned, we employ an LLM for open-set classification18.

  - As shown in Fig. 3, the LLM is provided with a list of object categories contained within each room cluster (e.g., "armchairs, carpet, table-lamp" for room-0)19191919.

  - The LLM analyzes these object compositions to infer the semantic room type (e.g., classifying room-0 as a "living room" and room-1 as a "bedroom")20.

  - This classification is performed dynamically at each high-level policy step as the scene evolves21.