## 3.4 Experiments

### Setup

The authors evaluated MoMa-LLM in both extensive simulation environments and a physical real-world deployment.

- Simulation: Experiments were conducted using the iGibson simulator, utilizing 15 interactive scenes based on real-world scans1. The agent controlled a Fetch robot to navigate and interact with these environments2.

- Real-World: The approach was transferred to a Toyota HSR robot equipped with an RGB-D camera and LiDAR3. The testing environment was an apartment comprising four rooms (kitchen, living room, hallway, and bathroom) with 54 object categories4444.

### Metrics

The evaluation employed standard metrics such as Success Rate (SR) and Success weighted by Path Length (SPL)5555. However, the authors strongly emphasized a novel metric designed to address limitations in existing evaluation paradigms:

- Area Under the Efficiency Curve (AUC-E): The authors argue that standard metrics rely on arbitrary time budgets (e.g., a maximum number of steps), which fail to distinguish between agents that search thoroughly but slowly versus those that are fast but less comprehensive6.

- Rationale: To resolve this, they calculate an efficiency curve plotting success rates against varying time budgets7. The AUC-E distills this curve into a single number, providing a robust measure of search efficiency that is independent of arbitrary cutoffs8888.

### Baselines

MoMa-LLM was compared against a diverse set of baselines ranging from heuristics to learning-based methods:

- Random & Greedy: Heuristic baselines that select actions uniformly or based on distance9.

- ESC-Interactive: An extension of the "Exploration with Soft Commonsense Constraints" approach, which scores frontiers based on object co-occurrences10101010.

- HIMOS: A hierarchical reinforcement learning approach adapted for interactive search11.

- Unstructured LLM: A baseline using the same LLM but provided with a raw JSON scene graph rather than the structured, language-grounded representation proposed by the authors12.

### Results

Simulation Results (Table I)

MoMa-LLM demonstrated superior performance across all metrics, proving to be the most efficient and effective method.

- Top Performance: MoMa-LLM achieved the highest Success Rate (97.7%), SPL (63.6), and AUC-E (87.2)13.

- Comparison: It significantly outperformed the Unstructured LLM (SR 86.3%, AUC-E 77.6%), highlighting that simply feeding a scene graph to an LLM is insufficient without structured grounding14. The Unstructured LLM also generated significantly more invalid actions (0.41 vs 0.19)15.

- Efficiency: While HIMOS achieved a high success rate (93.7%), it lacked efficiency (SPL 48.5) compared to MoMa-LLM16. ESC-Interactive performed well but struggled to optimize long-horizon sequences compared to the planning capabilities of the LLM17.

Real-World Results (Table III)

The system successfully transferred to the physical robot with consistent results.

- Success: Both MoMa-LLM and the strongest baseline, ESC, achieved an 80% success rate (8/10 episodes)18.

- Efficiency Gains: MoMa-LLM was drastically more efficient, traveling nearly half the distance of ESC (17.9m vs. 33.9m) and requiring fewer object interactions (2.2 vs. 3.5) to locate the target19.

- Conclusion: The results confirm that MoMa-LLM's structured knowledge representation enables more target-driven behavior, reducing unnecessary exploration and interaction compared to co-occurrence or heuristic methods20.

Would you like me to detail the "Fuzzy Search" tasks mentioned in the supplementary material, where the robot responds to abstract requests like "I am hungry"?